OLIVER TRAY SEALER

Models 1308-C and 1308-N

User’s operating and instruction manual

Section description 		 Page

DESCRIPTION/SPECIFICATIONS		5

GENERAL SAFETY INSTRUCTIONS		6

NAMEPLATES/PICTOGRAMS PART LIST		7

NAMEPLATES/PICTOGRAM LOCATION

BY ITEM NUMBER	8

BASIC MACHINE COMPONENTS AND SET UP		9

OPERATING PROCEDURES		11

ADJUSTMENT AND QUALITY		12

TECHNICAL SPECIFICATIONS		13

CLEANING AND MAINTENANCE		14

TROUBLE SHOOTING		17

WIRING DIAGRAM		18

DIAGRAM		19

PARTS LIST		20

MoDEL 1308 CARRIER		22

WARRANTY			23

WARRANTY PROCEDURE			24

Index

Description/Specifications

Description

The Oliver Model 1308 Heat Sealer is designed and manufactured to ensure a user friendly operation of producing film lidded trays with a cost effective approach.

The manually operated machine operates with a 120 V.A.C. outlet. The machine was designed to be compact to limit the amount of table space needed to operate.

Specifications

Space Requirements, (Shown with standard options)

Model 1308-C & 1308-N (All dimensions are shown approximate)

Shipping Weight – 65 lbs (30 Kg) (approximate)

Net Weight – 60 lbs (27 Kg) (approximate)

IT IS ESSENTIAL THAT ALL OPERATORS AND MAINTENANCE PERSONNEL OBSERVE THE FOLLOWING SAFETY PRECAUTIONS. IMPROPER INSTALLATION, MAINTENANCE, OR OPERATION OF THIS EQUIPMENT MAY CAUSE SERIOUS INJURY.

- Read this manual before attempting to operate your heat sealer. Never allow an untrained person to operate or service this unit.

- Observe all caution and warning labels affixed to the machine.

- Always unplug the machine before cleaning or servicing.

- Do not submerge in water. Do not hose down.

- Use original replacement parts.

6.	Wear proper, personal, protective, safety equipment if necessary.

7.	Keep hands away from moving parts of the machine while it is operating.

- In addition to these general safety instructions, please follow the more specific

safety instructions provided throughout the rest of this operating and instruction manual.

DO NOT USE FOR OTHER THAN ORIGINALLY INTENDED PURPOSE.

It is very important that the unit should be free of any unexpected energization, or be able to release hazardous energy during service or maintenance activities.

Therefore, always switch the machine off before unplugging the power cord from BOTH the unit and the wall.

WARNING

WARNING

General safety instructions

ITEM NO	                 PART DESCRIPTION	PART NUMBER

_______________________________________________________________________________________________________________________________________________________________

601		“OLIVER”	6401-5076

603		“STAY CLEAR”	6401-9043

604		“CONSULT OPERATOR...”	6400-3039

605	 	“ELECTRICAL SHOCK”	6400-5009

606		“UNPLUG BEFORE OPENING"	6402-1028

607		“HOT SURFACE”	6400-8021

**SEE PAGE 8 FOR NAMEPLATE LOCATIONS BY ITEM NUMBER

Nameplate/pictogram part list

(international models only)

Nameplate/pictogram locations by item number

Basic machine components and set up

FILM SUPPLIED BY OLIVER PRODUCTS COMPANY IS WOUND WITH THE ADHESIVE SIDE IN.

FEED FILM FROM THE TOP OF THE ROLL AS SHOWN.

NOTE

Set up (continued)

CAUTION

- Plug the power cord into a properly grounded outlet. Avoid the use of extension cords.

- Pusher rocker switch to “On” position (red light indicates on).

NOTE: It should take approximately 15 minutes for the machine to warm up to operating temperature.

- Place tray carrier in its designated area with the dowel pins facing down

- Place filled tray in tray carrier. Make sure the food in the tray is not higher than the flange of the tray

- Grasp the corners of the film and pull it straight across the tray going a 1⁄4 inch past the flange of the tray

- Pull handle down with both hands until it stops. Hold the handle down for approximately 2 seconds.

- Lift the handle to its upright position.

- Remove tray from tray carrier.

- Repeat steps 4 thru 8.

IT IS NOT RECOMMENDED TO LEAVE THE MACHINE ON WHEN IT WILL BE OUT OF OPERATION FOR AN EXTENDED PERIOD OF TIME.

Operating procedures

Your OLIVER Model 1308 was tested at the factory using Oliver trays and film.

- THE TEMPERATURE CANNOT BE ADJUSTED BY THE OPERATOR

The heater platen temperature is factory set at approximately 300°F (Approx. 150°C). Contact the Oliver Service Department if you suspect the heated platen temperature is not correct.

It is necessary to periodically check for a proper seal. This can be done by pull the film over an empty tray carrier, sealing it like normal and peeling the film. If there is a proper seal, the pattern on the film should be a complete outline of gasket material provided.

A poor seal will occur if the product in the tray contaminates the tray flange or the pressure generated by the sealing mechanism is less than 15lbs. The pressure can be

adjusted by removing the acorn nut atop the center arm and giving a few turns to the nut on the heater adjustment bolt. This will expose more threads between the casting and the sealing mechanism, therefore, generating more pressure.

Under different operating conditions, it may be necessary to increase or decrease the sealing time. This is done by varying the length of time the operating handle is in the down position. Holding the handle down longer makes the seals stronger. Although a stronger seal may cause shredding or tearing of the film when removing film from tray.

NOTE

Adjustment and quality

Technical specifications

- Temperature Range:	Factory preset to approx. 300°F/150°C

- Weight:	60 lbs (27 kg)

- Electrical:	120 volts AC (USA) 4 Amps (3.4A drawn),

60 Hz

- Finish:	White Powder Coat / Stainless steel

- Max roll diameter:	5-1/2’’ COD

- Max roll width:	9’’

- Std. seal area:	9’’ x 6.375’’

- Max seal area:	9’’ x 7.5’’

- Std. gaskets:	White/60 duro/ approximately 1/8” seal

- Handle pull down: 	15lb to 20lb

- Dielectric breakdown:	1250 for one second

- Constructed of:	Cast Aluminum/Stainless steel

- These cleaning recommendations are not meant to replace plant standard manufacturing procedures or regulatory requirements.

- If the machine has been in operation, allow the unit to cool before cleaning.

- Do not submerge unit in water. Do not wash down.

- Tray carrier is NOT recommended for automatic dishwashers.

- CAUTION HOT: The heated platen is HOT! Caution must be used to prevent injury from heated platen.

- Make sure the unit has been unplugged for a minimum one hour before cleaning.

- It is important that the surface of the heater platen is kept clean and smooth. Regularly clean the OLIVER Model 1308 with a mild cleaner and a damp cloth. If food product comes in contact with the surface of the platen, it tends to burn on and become hard. This results in an irregular surface on the face of the platen which can result in poor seals. If this happens, it will be necessary to remove this contaminate material. When cleaning care must be taken to avoid scratching or gouging the surface of the platen. DO NOT SCRAPE THE SURFACE OF THE PLATEN WITH SHARP OBJECTS AND AVOID THE USE OF METAL TOOLS. The use of plastic or soft-metal scouring pads such as SCOTCH BRITE™ or CHORE BOY® provides a safe and effective means of cleaning the platen. Be sure to wipe all surfaces with a sanitizing agent after cleaning.

- IF CLEANING THE HEATER PLATEN WHILE THE UNIT IS STILL ON, MAKE SURE THAT HAND PROTECTION IS WORN AT ALL TIMES TO PREVENT SKIN CONTACT.

NOTE

CAUTION

WARNING

CAUTION

Cleaning and maintenance

CLEANING PROCEDURES

- Unplug the unit and allow one hour for cooling.

- Remove tray carrier.

- Place tray carrier in the dishwasher or wipe down with a standard cleaning solution.

- Lift operating handle to expose the bottom side of the heater, and then wipe off the heater platen.

- Use a sanitizing solution to spray and wipe down the rest of the unit.

- Replace tray carrier

THE DELRIN LINKS LOCATED ATOP THE UNIT SHOULD BE LUBRICATED WITH A FOOD

GRADE MATERIAL ONCE A MONTH TO PREVENT WEARING.

CLEANING THE CUTTER BLADE

- Lift handle up to its resting position.

- Use a damp cloth with sanitizing solution to wipe off any access food particles located on the back of the cutter blade.

THE CUTTER BLADE IS SHARP. USE A PROTECTIVE GLOVE TO DO THIS CLEANING OPERATION. FAILURE TO DO SO MAY RESULT IN SERIOUS INJURY.

NOTE

CAUTION

Cleaning and maintenance (continued)

REPLACING AND CHECKING GASKETS

- If there are any gouges, cuts or gaps in the gasket, the gasket will have to be replaced.

- Be sure to replace all the gaskets not just the piece that is damaged.

  - When reinstalling gaskets, the gasket must be seated all the way down in thegroove.

- Do not stretch gasket when reinstalling.

- After the new gaskets are installed, place the tray carrier upside down on a flat surface and check to see if the newly installed gasket is even.

- If not even, depress or lift gasket to provide an even heat sealing surface.

- It is necessary to periodically check for a proper seal. This can be done by pull the film over the empty tray carrier, sealing it like normal and pulling the film. If there is a proper seal, the pattern on the film should be a complete outline of the edges of the tray.

DISPOSAL PROCEDURE

When considering disposing of your unit, please contact local authorities for any special instructions.

Cleaning and maintenance (continued)

SYMPTOM	CORRECTIVE ACTION

#### Poor seal quality	Verify that the platen is heated and is at the 	correct temperature (approximately 300°F/150°C)

Check to see if the platen is dirty

Check rubber gasket on tray holder for

damaged or missing pieces

_______________________________________________________________________

Platen does not heat up	Verify that the power cord is plugged into outlet

Verify that the power switch is in the “on“ position

Verify that there is power at the outlet

_______________________________________________________________________

Film sticking to heater 	Film is coming off the roll the wrong direction or

improper threading (feed from top).

_______________________________________________________________________

Shredding film	Seal time or pressure needs to be adjusted.

Trouble shooting

Wiring diagram

1-60-120V

REF. W/D 1308C12002

1-60-230V

REF. W/D 1308C12000

Diagram

Type to enter text

ITEM NO	PART DESCRIPTION	PART NUMBER

001	Base	1308-0015-301

002	Bumper-Rubber	5902-0035

004	Tube-Aluminum 7/8” O.D.	4639-1414-1116

005	Bearing-Roll End	5252-3002

006	Guide-Roller Film	1908-0041

007	O-ring	6909-3211

008	Rod-Film Dispensing	1308-0018

010	Tube-Film Shaft	1308-0016

012	Roller-Pinch 	1308-0048-1

013	Spacer-Pinch Roller	1308-0049-1

101	Arm-Center	1308-0014

102	Shaft-Center Arm Link	1308-0001

103	Bearing-Bronze Shaft	5254-3215

104	Clip-E Style (3/4” Shaft)	5840-2841

105	Link-Center Arm	5502-1900

106	Shaft-Mid Pivott	1308-0002

109	Weight-Counter	1308-0042

110	Collar-White Half 	1308-0501

111	Screw-Hex Socket Head 1⁄4-28 x 5/88	5725-9500

201	Assembly-Handle	1308-0500-1

202	Shaft-Handle Pivot	1308-0007-001

Parts list

301	Platen-120V Heater	1308-0061

301	Platen-230V Heater	1308-0061-002

302	Spring-Compression 	7015-2200

303	Blade-Cutting	1308-0011-1

304	Plate-Cutting Blade Mounting	1308-0056

305	Spacer-Blade Mounting	1308-0024-001

307	Screw-Shoulder 1/4 x 3/8	5842-8997

310	Spring-Compression 2”	7014-4203

311	Channel-Spring Backer	1308-0055

312	Screw-Shoulder 1⁄4-28 x 3/4	4560-0908-1201

313	Cover-Heater	1308-0058

314	Bar-Pusher	1308-0057

315	Bolt-Carriage 1⁄2-13 x 4-1/2”	5804-1722

320	Box-Relief	1308-0059

321	Bushing-Strain Relief	5765-1110

322	Nut-Hex Head Lock (PG-11)	5766-7786

327	Enclosure-Electrical	1308-0060-2

329	Brace-Ground	1308-0062

333	Inlet-Fused Power w/ Switch (1 pole)	5746-7910

333	Inlet-Fused Power w/ Switch (2 pole)	5746-7911

334*	Fuse-5 x 20 mm (4Amp)	5725-9571

334*	Fuse-5 x 20 mm (2Amp)	5725-9500

335*	Drawer-Fuse (1 pole)	5746-7950

335*	Drawer-Fuse (2 pole)	5746-7951

- Item Not Shown

Bold items are required for 120V North American sale

Parts list (continued)

MODEL 1308 TRAY CARRIER PARTS LIST

ITEM NO	PART DESCRIPTION	PART NUMBER

701*		Carrier-Tray 3 Compartment	1308-0013-0003

702		Pin-Dowel 3/16” x 1”	5835-6471

703**	Cushion-White Silicone Extruded 	6516-0028

704		Nameplate-“US Patent 5,791,120”	6402-1014

- Tray carries do not come with stock units. They are ordered separate and customized based on the tray that is being used in the sealing operation.

** Cushions differ from tray carrier to tray carrier. The cushion that is used is determined by the engineer during the design process.

FOR SERVICE PARTS CALL SOLPAK AT 1-877-476-5725

Model 1308 carrier

PARTS

Oliver Products Company (Oliver) warrants that if any part of the equipment (other than a part not manufactured by Oliver) proves to be defective (as defined below) within one year after shipment, and if Buyer returns the defective part to Oliver within one year, Freight Prepaid to Oliver’s plant in Grand Rapids, MI, then Oliver, shall, at Oliver’s option, either repair or replace the defective part, at Oliver’s expense.

LABOR

Oliver further warrants that equipment properly installed in accordance with our special instructions, which proves to be defective in material or workmanship under normal use within one (1) year from installation or one (1) year and three (3) months from actual shipment date, whichever date comes first, will be repaired by Oliver or an Oliver Authorized Service Dealer, in accordance with Oliver’s published Service Schedule.

For purposes of this warranty, a defective part or defective equipment is a part or equipment which is found by Oliver to have been defective in materials workmanship, if the defect materially impairs the value of the equipment to Buyer. Oliver has no obligation as to parts or components not manufactured by Oliver, but Oliver assigns to Buyer any warranties made to Oliver by the manufacturer thereof.

This warranty does not apply to:

- Damage caused by shipping or accident.

- Damage resulting from improper installation or alteration.

- Equipment misused, abused, altered, not maintained on a regular basis, operated carelessly,or used in abnormal conditions.

- Equipment used in conjunction with products of other manufacturers unless such use is approved by Oliver Products in writing.

- Periodic maintenance of equipment, including but not limited to lubrication, replacement of wear items, and other adjustments required due to installation, set up, or normal wear.

6. Losses or damage resulting from malfunction.

The foregoing warranty is in lieu of all other warranties expressed or implied AND OLIVER MAKES NO WARRANTY OF MERCHANTABILITY OR FITNESS FOR PURPOSE REGARDING THE EQUIPMENT COVERED BY THIS WARRANTY. Oliver neither assumes nor authorizes any person to assume for it any other obligations or liability in connection with said equipment. OLIVER SHALL NOT BE LIABLE FOR LOSS OF TIME, INCONVENIENCE, COMMERCIAL LOSS, INCIDENTAL OR CONSEQUENTIAL DAMAGES.

Warranty

Warranty procedure

- If a problem should occur, call SOLPAK at 1-877-476-5725 ext. 221

- A Solpak partner will determine if the warranty will apply to this particular problem and inform you of the actions to take (technician, exchange,…).

- A replacement 1308 can be made available to you in the event your unit has to be repaired in our service centre.

The Solpak team

thanks you for your trust!